# EE49001: Control and Electronic System Design

Assignment-3: Determination of DC Gain, Part:1

Submitted By:

21EE30004: Anirvan Krishna | 21EE30001: Aditya Kumar

From the above block diagram

Gs=Gcs⋅Gps
=6Ks+1s+2s+3

# Design K such that gain crossover frequency is 2 rad/s

## Theoretically

From the above-mentioned transfer function, the gain equation is

Gs=K1+ω12⋅1+ω22⋅1+ω32

For gain crossover frequency to be ωc to be 2 rad/s

20logGjωc=0
⇒Gjωc=1
⇒K=1+22⋅1+12⋅1+232
⇒K=1309≈3.8006

## Design Steps

If K=1 is used in the transfer function G the following Bode plot is obtained

From the phase plot of Gs it is evident that ∠Gjωc=-142.125°. Thus ∠Gj2+180°>0 and it can be concluded that gain crossover frequency of 2 rad/s can be achieved.

As can be observed, 20logGj2=-11.597dB, thus to intersect the 0 dB line at ω=2rad/s the magnitude plot must be pulled down by 11.597dB.

Therefore, our required DC gain is

K=1011.59720≈3.8005

Which is very close to our calculated value in the first step.

# Design K such that Phase Margin is 60deg

## Theoretically

∠Gjω=-tan-1ω-tan-1ω2-tan-1ω3=PM-180
⇒tan-1ωc+tan-1ωc2+tan-1ωc3=120°
⇒ωc≈1.505rad/s

Therefore,

Gjωc=1
⇒k=1+1.5052⋅1+1.50522⋅1+1.50532
⇒k≈2.53

## Design steps

From the phase plot it can be seen that ωc=1.505 for ∠Gjωc=-120°

As can be observed from the magnitude plot that the magnitude plot needs to be pulled down by 8.0624dB so that it intersects 0dB line at ωc.

Therefore, our required DC gain is

K=108.062420≈2.528

# Design K such that Position Error Constant is 5

## Theoretically

For a unity feedback system, as is given, the position error constant can be defined as

kp=lims→0Gs
=lims→06Ks+1s+2s+3=5
⇒K=5

## Design Steps

Therefore, from the above magnitude plot the it can be observed that at the low frequency end, where the plot is flat, it is lower than the desired kp by 13.9794dB (since the magnitude plot is flat at 0dB). Thus, the plot needs to be shifted up by 13.9794dB. Hence our required DC gain will be

K=1013.979420
≈5

# Design K such that 20logGs≥10dB for ω∈0,0.3

## Theoretically

Since it is sufficient to make 20logGj0.3≥10 for the said condition to hold. Therefore,

Gj0.3=10

⇒K1+0.312⋅1+0.322⋅1+0.332=10

⇒K=10⋅1.09⋅1.0225⋅1.01

K ≈3.3551

## Design Steps

Since Gj0.3=-0.514, thus to bring Gj0.3 to 10dB, we need to shift the magnitude plot up by 10--0.51411=10.51411. Therefore, our required DC gain is

K=1010.5141120
≈3.354

# Desired Plots

## Position Error Constant is 5

K is set to 5 as obtained before in our calculation and the bode plot of Gs is obtained.

As can be observed the gain at lower frequencies (assumed 0.02) is around 13.979 with is approximately equal to 20log5. Thus, it can be concluded that the position error constant Kp is 5.

## Phase Margin is 60deg

K is set to 2.528 as obtained before in our calculation and the bode plot of Gs is obtained.

As can be observed from the above bode plot, the gain crossover frequency is 1.505 rad/s and correspondingly ∠Gj1.505=-120deg. Thus, the phase margin will be -120+180=60° Therefore, design of K is just as expected.

## Gain at 0.3 is greater than 10dB

K is set to 3.354 as obtained before our calculation and the bode plot of Gs is obtained.

As can be observed that the gain at 0.3 rad/s is 10 dB which is consistent to our design goals.